# PART 1 	GENERAL

# SECTION INCLUDES

      - Section 07 72 00 - Plastic Roof Vents.

# RELATED SECTIONS

      - Section 07 31 13 - Asphalt Shingles

# REFERENCES

      - CAN3 A93-M82 (R2003) - Natural Airflow Ventilators for Buildings.

      - CRCA (Canadian Roofing Contractors' Association)

      - NRCA (National Roofing Contractors' Association)

# SUBMITTALS FOR REVIEW

      - Section 01 33 00: Submission procedures.

      - Product Data: Provide component sizes, rough-in size requirements, and finishes.

      - Shop Drawings:  Indicate dimensions, weights, and placement of openings and holes

# SUBMITTALS FOR INFORMATION

      - Section 01 33 00: Submission procedures.

      - Installation Data:  Manufacturer's special installation requirements.

# QUALITY ASSURANCE

      - Manufacturer Qualifications:  Company specializing in manufacturing the Products specified in this section with minimum [three (3)] [	   ] years experience.

# DELIVERY, STORAGE, AND PROTECTION

      - Section 01 61 00: Transport, handle, store, and protect products.

      - Inspect for damage.

# WARRANTY

      - Section 01 78 10:  Warranties.

      - Manufacturer’s Limited Warranty: warrants products to be free of defects for a period of 10 years when installed in accordance with the manufacturer’s written instructions.

      - Contractor's Warranty:  Provide [________  (    )] year [warranty] on roofing, dated from time of Substantial Completion.   Not all provincial associations use the term "Certificate of Assurance"; edit accordingly.

# Part 2	Products

# MANUFACTURERS

      - Menzies Enterprises Ltd. dba Menzies Metal Products.

Direct Phone:  604-530-0712

Toll Free Phone:  800-665-8840 Direct Fax: 604-530-8482

Web:  www.menzies-metal.com

Email:  sales@menzies-metal.com

      - Substitutions:  [Not permitted] [Refer to Section 01 61 00].

# MATERIALS

      - HD Polyethylene.

# ROOF VENTS

      - Roof Vents: Menzies SV50 Plastic Roof Vent, CAN3-A93 Type B, high impact, UV resistant plastic vent.

          - Opening Size:  178 x 250 mm (7 x 10 inch).

          - Free Area:  323 sq cm (50 sq in).

          - Color:  [Black] [Dark Brown] [Light Brown] [Grey] [Wood Grey] [White].

# Part 3	Execution

# PREPARATION

      - Menzies Verify shapes, dimensions and slope of surfaces to be covered before manufacture.

      - Coordinate vent installation with adjacent material installation.

# INSTALLATION

      - Install roof vents to manufacturer's written instructions.

      - Anchor vents securely in place, using fasteners, sealants, and other miscellaneous items as required to complete venting systems.

      - Install level, plumb, true to line and elevation.

      - Provide uniform, neat seams with minimum exposure of sealant.

      - Install vents to fit substrates and provide watertight performance.

# Cleaning

      - Do not use cleaning solutions, tools or techniques that may impact the performance of the product or the roofing system.

# END OF SECTION