## Koala Deck Drain No Hub PVC

## INSTALLATION INSTRUCTIONS

- Cut an appropriate sized hole over the receiving drainpipe.

- Remove any debris or material that may prevent the drain flange from laying flush on the deck.

- Remove the strainer from the drain.

- The flange must lay flush on deck. Secure the drain to the deck with a minimum of 3 broad-head fasteners.

- Make certain decking material reaches the edge of drain bowl but does not go into the drain bowl.

- Apply and secure the roofing material according to the manufacturer’s instructions.

- Replace the strainer and tighten securely.

- Under deck connection is by others.

- Adhere to installation requirements as determined by the regulatory authorities for your region.